| Motor size | 1.7” flange – 42 mm square | Step Angle | 1.8° ±5% |
| --- | --- | --- | --- |
| Front Shaft | 17.00 mm length – Ø 5.00 mm with D-cut 12.5 x 1.0 mm | Rear Shaft | None |
| Winding | # 4 lead wires – UL3266 AWG#26 | Lead Wire Length | 300 mm ±10 with connector on board |
| Rated Voltage | 4.10 Volt | Rated Current | 1.00 A |
| Resistance | 4.10 ohm ±10% | Inductance | 9.00 mH ±20% |
| Winding | Bipolar parallel | Holding Torque | 0.45 Nm min. |
| Rotor Inertia | 69 g.cm² | Insulation class | Class B, 130°C |
| Ambient temperature | -20° ~ +50° C | Temperature rise | 80K max |
| Shaft thrust load | 10 N | Shaft radial load | 21 N (at front shaft end) |

Mechanical drawing
(Dimensions in mm)